|  | EUROPEAN COMMISSION DIRECTORATE-GENERAL ENVIRONMENT Directorate D - Water, Chemicals & Cohesion ENV.D.2 - Water and Marine |
| --- | --- |

Brussels, 21 January 2008

Document ENV-COM240108-5

Water Framework Directive Committee meeting 24 January 2008

Agenda item 5: intercalibration

On 27 September 2007 DG Environment sent a letter to the Water Directors requesting further clarification on the degree of comparability between some specific results of the intercalibration exercise. A reservation was held on the inclusion of these results in the annex of the draft Commission Decision publishing the intercalibration results.

The issue was discussed at the Ecostat meeting in October 2007 and at the WFD Committee meeting on 8 November 2007. A questionnaire with a template for calculation of the comparability of the Member States’ results was sent to GIG coordinators to compile the responses in a harmonised manner.

The attached paper presents an analysis of the information provided by the GIGs, based on selected comparability indicators. Some cut-off values for those indicators are proposed and used to make an assessment of the comparability of the various GIG results. The paper is a first draft and will be distributed to the Ecostat experts for comments.

On the basis of the attached analysis, and subject to further checking of the results, it is proposed to withdraw from the annex of the draft Commission Decision the following results:

- Lakes Central-Baltic macrophytes and phytoplankton composition

- Coastal Baltic macroalgae (as requested by SE and FI representatives)

- Coastal Mediterranean macroalgae

In addition, DG Environment maintains its reservation on the following results:

- Coastal North-East Atlantic macroinvertebrates: despite excellent level of comparability achieved among 8 countries, further investigation is on-going on the interpretation of the normative definitions, the boundary setting and the comparison of the Belgian/Dutch method with the rest.

- Coastal Mediterranean angiosperms: the information provided is not clear enough and further investigation is on-going.

The Committee is invited to:

- Take note and discuss the document and the proposals

- Send comments in writing to Jorge Rodriguez Romero (Jorge.Rodriguez-Romero@ec.europa.eu) by 15 February

Status box Title : Comparability of the results of the intercalibration exercise - summary of responses and way forward Version no.: 1.0 Date: 21 January 2008 Status of the document : Author(s): Wouter van de Bund, Sandra Poikane, Jorge Rodriguez Romero Summary: On 27 September 2007 DGENV sent a letter to the Water Directors requesting further clarification on the degree of comparability between some specific results of the intercalibration exercise. DGENV indicated that they hold a reservation regarding the inclusion of these results in the annex of the draft Commission Decision. A questionnaire with a template for calculation of the comparability of the Member States’ results was sent to GIG coordinators to compile the responses in a harmonised manner. This paper is presenting an analysis of the results and is summarising the comparability of the GIG results.

Comparability of the results of the intercalibration exercise - summary of responses and proposed way forward

# Introduction

On 27 September 2007 DGENV sent a letter to the Water Directors requesting further clarification on the degree of comparability of the intercalibration results. Two specific questions were raised:

- Firstly, if the same monitoring and classification system is applied in two or more member States, why the boundaries of good ecological status are different.

- Secondly, the level of comparability was too variable between GIGs. There should be common criteria to assess the level of agreement between different methods of the member States.

As suggested in the letter, at their meeting in October 2007 the ECOSTAT experts considered the possibilities to solve or explain these issues in the short term. It was agreed that each concerned GIG would provide further clarification. A questionnaire with a template for calculation of the comparability of the Member States’ results was sent to GIG coordinators to compile the responses in a harmonised manner. Most of the GIGs concerned have provided this information for most of the quality elements. This paper summarises and analyses the responses of the GIGs, and presents proposals for indicators and criteria to agree on sufficient comparability between the results of the Member States’ classifications.

# First issue: Countries using the same classification system but reporting different boundaries

The GIGs were asked to indicate to what extent the following factors are contributing to the differences in the national boundaries for the same classification metric:

  - Typology differences: GIGs argued that the criteria for the intercalibration type parameter boundaries have very wide ranges, and may include several national types. As a consequence national reference conditions may be slightly different, resulting in different class boundaries between the Member States, even if they use the same classification metric.

  - Differences between the methods themselves: It was noted that there are considerable uncertainties in all basic aspects of a classification systems (monitoring systems; reference conditions; classification). Due to this uncertainty a ‘band of acceptability’ was introduced and set at +/- 5% to allow compensating for such inherent variability between the methods. This uncertainty is applicable to all MS methods (whether they use the same metrics or not).

  - Limitations in national dataset used to derive the national boundaries (i.e. limited reference condition sites, etc.): The number of reference sites was often very low, causing considerable uncertainty in reference values which is likely to cause some differences between Member States’ classification results.

  - Different national views of the boundaries (including differences in the procedures used to set national reference conditions and boundaries): Some of the GIGs could not exclude that this factor has played a role, but they argue that these differences, if they exist, are so small that they fall within the ‘band of acceptability’.

In conclusion, the main justification for the different classification values between Member States was that even if the classification metrics may be the same, the uncertainly in the various steps of monitoring and classification as well as in  the intercalibration is inevitably  resulting in slightly different EQR boundaries. The ‘band of acceptability’ of 5% is considered the best that could be achieved given the current limitations. Currently all results fall within this band.

It is also argued that it would be possible to reduce the uncertainty by defining the reference conditions more precisely and by using more harmonised monitoring methods.

# Second issue: Differences between GIGs in criteria for comparability

The main problem for these results is that the GIGs used different criteria to evaluate whether or not the assessment results were comparable, making it very difficult to judge whether the intercalibration exercise has achieved the same level of comparability for all results.

In response to the request by DG Environment, the GIGs have re-analysed their data, calculating a number of common comparability metrics. After reviewing the information it was decided to focus on three of those – the absolute average class difference, the percentage of agreement using three classes, and the percentage of agreement using five classes (see Annex for details). Based on these criteria, the following conclusions are drawn:

- The main criterion recommended to be used is the absolute average class difference that shows to which extent the Member States’ methods may give different classification result. It is proposed to use a class difference of less than a half class (0.5) as a criterion for sufficient comparability.

- The second criterion proposed to be used is the percentage of agreement between Member States’ classification methods. This can be used mainly as supporting information, since value is very sensitive for how data points are distributed over the quality range, and thus it needs to be considered with some caution.

- Based on these two criteria it seems that in most of the GIGs the assessment methods are sufficiently comparable between the Member States.

- However, there are some cases that still show a very low comparability, suggesting that further harmonisation would be needed.

- There were also GIGs that were not able to provide data nor any clear explanation or the comparability.

# Conclusions and proposed way forward

For the first group (countries using the same classification system reporting different boundaries) the GIGs have provided explanations of the reasons behind the differences between Member States. The occurrence of these differences stress a need for further work on improving comparability between the national monitoring and assessment methods in the future, especially in order to reduce the uncertainties in the intercalibration methods. One of the key issues is to develop and agree on more harmonised and precise procedures for setting reference conditions.

This concerns the following GIGs/countries:

| GIG | Quality element | Countries affected | Sufficient comparability demonstrated? |
| --- | --- | --- | --- |
| Rivers Central-Baltic | Macroinvertebrates | BE(W), FR, LU | Yes |
| Rivers Central-Baltic | Phytobenthos | BE(W), EE, LU, SE | Yes |
| Rivers Mediterranean | Macroinvertebrates | EL, IT, CY | Yes |
| Rivers Mediterranean | Phytobenthos | PT, ES | Yes |
| Rivers Northern | Phytobenthos | FI, SE | Yes |
| Coast North-East Atlantic | Macroinvertebrates | FR, DE, ES | Yes |

For the second group of results (differences between GIGs in criteria for comparability) it has been possible to demonstrate sufficient comparability for most of the GIGs and biological quality elements:

| GIG | Quality element | Countries affected | Sufficient comparability demonstrated? |
| --- | --- | --- | --- |
| Lake Alpine | Macrophytes | AT, DE | Yes |
| Lake Northern | Macrophytes | FI vs NO, UK, SE, IE | Yes |
| Lakes Central/Baltic | Macrophytes | BE, DE, EE, LV, NL, UK | No |
| Lakes Central/Baltic | Phytoplankton composition | BE, DE, EE, FR, HU, NL, UK, IE | No |
| Coast Baltic | Macroinvertebrates | SE, FI, DK, DE | Yes, except DE and type B2 |
| Coast Mediterranean | Macroinvertebrates | GR, CY, ES, SI | Yes |
| Coast North-East Atlantic (NEA) | Macroinvertebrates | DK, FR, ES, NO, PT, UK, IE, DE, NL, BE | Yes, except for BE/NL method |
| Coast Baltic | Macroalgae | SE, FI | Not enough information |
| Coast Mediterranean | Macroalgae | GR, ES, SI, FR, CY | No |
| Coast Mediterranean | Angiosperms | FR, IT, MT, ES, GR | Not enough information |

The comparability of Coast Baltic Macroinvertebrates IC Common type B2, Coast Mediterranean Macroalgae, Lakes Central/Baltic Macrophytes and Lakes Central/Baltic Phytoplankton composition results is not considered sufficient and it is proposed to ask the GIGs to continue working on improving comparability within the intercalibration work programme 2008-2011.

For the Coast Baltic Macroalgae and Mediterranean Angiosperms no data could be provided in this stage, so no conclusions could be drawn on the comparability.

# Annex:

# Analysis of the comparability of the Intercalibration Option 3 results - Summary and evaluation

Introduction

Following the request of the DG ENV (Letter to the EU Water Directors dated 27.09.2007), five Geographical Intercalibration Groups (GIGs), provided classification results to carry out the Option 3 performance comparison for seven biological quality elements (see Table 1):

  - Coastal Baltic GIG provided several bilateral comparisons of macroinvertebrate methods between Sweden and Finland for the common Intercalibration types B0, B2 and B3, and between Sweden and Denmark for type B12;

  - Lake Alpine GIG provide results separately for two Common Intercalibration types – LAL3 and LAL4;

  - Lake Central Baltic GIG provided results separately for two Intercalibration types – LCB1 and LCB2 for macrophyte assessment methods;

  - Coast North-East Atlantic (NEA) GIG provided two different sets of results for the Benthic macroinvertebrates: one including 5 methods and 7 Member States (DK, FR, ES, NO, PT, UK, IE). The another set  of results incorporated also the German macroinvertebrate assessment method in the comparisons;

  - The Coast Mediterranean GIG compared 2 Macroalgae classification methods (Benthos and EEI) between 2 countries (Greece using EEI; and Spain using Benthos); and 3 macroinvertebrate methods used by four Member States:  Slovenian M-AMBI, Greece and Cyprus Bentix, and Spain Medocc.

.

Table 1. Intercalibration Option 3 results:  GIGs,  BQEs, participating Member States (MS) and Intercalibration (IC) common types. When ever two MS share the same method, those are linked with a hyphen.

| Geographical Intercalibration Group (GIG) | Biological Quality Element (BQE) | Participating MS and IC types |
| --- | --- | --- |
| Coastal Baltic GIG | Benthic invertebrates | SE-FI: IC types B0, B2, B3 SE-DK: IC type B12 |
| Coastal North East Atlantic (NEA) GIG | Benthic invertebrates | Comparison in 2 versions: 5 methods, 7 MS: DK, FR-ES , NO, PT, UK-IE 6 methods, 8 MS: DE, DK, FR-ES, NO, PT,UK-IE |
| Coastal Mediterranean GIG | Macroalgae | 2 methods, 2 MS (BENTHOS = ES, EEI = GR/EEI) |
|  | Benthic invertebrates | 3 methods, 4 MS: GR-CY, ES, SI |
| Lake Alpine GIG | Macrophytes | AT and DE: 2 types separately LAL3 and LAL4 |
| Lake Central Baltic GIG | Macrophytes | BE, DE, EE, LV, NL, UK: 2 types separately LCB1 and LCB2 |
|  | Phytoplankton | BE, DE, EE, FR, HU, NL, UK-IE |

3 GIGs have not provided the results for the following reason (see Table 2)

Table 2. Intercalibration Option 3 results:  GIGs,  BQEs, participating Member States (MS)  which have not provided the results.

| GIG | Quality element | Countries affected | Explanation |
| --- | --- | --- | --- |
| Lake Northern | Macrophytes | FI vs NO, UK, SE, IE | Finland decided to withdraw their method from the Decision Other MS have used Option 2 for the comparison, so no more explanations needed |
| Coast Baltic | Macroalgae | SE, FIN | GIG has not provided any data for comparison, as they have not collected a joint data base. |
| Coast Mediterranean | Angiosperms | FR, IT, MT, ES, GR | GIG has not provided any data for comparison, explaining that Option 2 and Option 3 hybrid was used Additional explanations needed |

Results

Several indicators were used to evaluate Intercalibration Option 3 performance and degree of comparability of the assessment systems.

1. Absolute average class difference

The main criterion recommended to be used is the absolute average class difference that shows to which extent the Member States’ methods may give different classification result. The possible criterion for sufficient comparability is proposed to be less than a half class (0.5) difference.

This indicator reflects to which extent classification systems give different results (but not indicating whether one or more systems appear to be more or less precautionary than the others). Differences between classification systems can be due to systematic differences and/or due to random error. The smaller the difference – the better comparability there is between the systems:

- 1.0 class difference indicates that system A results on average one class different assessment comparing with other systems;

- 0.5 class difference indicates that system A assessment results at 50% of the cases into 1 class difference in the assessment comparing with other systems.

Justification for the comparability indicator:

- It enables to evaluate a  total difference between 2 or more classification systems;

- This indicator incorporates also the cases of “strong misclassification” -  when the results differ by two or more quality classes;

- It is not sensitive to the data distribution over the quality range (a significant benefit comparing with the other indicators, e.g. % agreement of classifications using 3 or 5 quality classes).

Table 3 Intercalibration Option 3 results: Absolute average class difference of the GIG methods comparisons, arranged in increasing order.

| Abbreviation | Geographical Intercalibration group, Biological Quality Element and IC type (where appropriate) | Absolute average class difference (in classes) |
| --- | --- | --- |
| L Alp Macrophy 4 | Lake Alpine Macrophytes Type LAL4 | 0.29 |
| C NEA Benthic 5 | Coastal North East Atlantic GIG Benthic invertebrates (5 methods) | 0.32 |
| C NEA Benthic 6 | Coastal North East Atlantic GIG Benthic invertebrates (6 methods) | 0.35 |
| C BAL Bent SE-DK | Coastal Baltic GIG (Sweden-Denmark) | 0.36 |
| C BAL Bent SE-FI B0 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B0 | 0.36 |
| C BAL Bent SE-FI B3 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B3 | 0.39 |
| C MED Bent | Coastal Mediterranean GIG Benthic invertebrates | 0.43 |
| L Alp Macrophy 3 | Lake Alpine Macrophytes Type LAL3 | 0.49 |
| C BAL Bent SE-FI B2 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B2 | 0.54 |
| C MED Macroalgae | Coastal Mediterranean GIG Macroalgae | 0.58 |
| L CB Phyto | Lake Central Baltic GIG Phytoplankton | 0.79 |
| L CB Macrophy 1 | Lake Central Baltic GIG Macrophytes Type LCB1 | 0.88 |
| L CB Macrophy 2 | Lake Central Baltic GIG Macrophytes Type LCB2 | 0.90 |

The results of the absolute average class difference

- Range from 0.29 class difference (Lake Alpine GIG macrophyte comparison for IC type LAL4) to 0.90 class difference (Lake Central Baltic comparison of macrophyte assessment methods);

- Most of the results occur in range from 0.3 to 0.6 class difference, roughly corresponding to 40 - 70% agreement between the classification systems (see Fig 3);

- The possible criterion for sufficient comparability is proposed to be less than a half class (0.5) difference (so the results above 0.5 class difference are considered as not satisfactory).

Figure 1 . Intercalibration Option 3 results: Absolute average class difference of the GIG methods comparisons, arranged in increasing order. The possible criterion for sufficient comparability is proposed to be less than a half class (0.5) difference (red line).

2. Level of Agreement using 3 classes

The second criteria proposed to be used in the evaluation is the percentage of agreement between Member States’ classification methods. This can be used only as a supporting criteria, since these values are highly sensitive for the data distribution over the quality range, and thus need to be considered with caution.

- This indicator reflects a degree of consensus (level of agreement) between two or more assessment systems expressed as the percentage of cases where two methods give the same class classification result;

- The assessment of comparability is focused to the upper range of the classification results, as it is most important to demonstrate that different methods show good correspondence for high-good and good-moderate boundaries, in line with the intercalibration objectives.

- In this reasoning, two methods are considered to be sufficiently comparable if those give compliant results for high, and good, and including all the results from moderate/poor/bad status classes in one block (see Fig. 2 for an example, yellow zone illustrates the area where classifications are considered to give the same results).

- The main advantage of this indicator is that it focuses on agreement between methods on the HG and GM boundaries, and that possible disagreements in the lower quality classes do not affect the results.

- A drawback if this indicator is that the outcome highly depends on how the data is distributed over the quality range. If there are many data points in the low quality range the level of agreement tends to be high. Because of this, it is not possible to directly compare results between GIGs if the distribution of the data is very different.

| Version A: data distributed in EQR range from 0 to 1: as consequence agreement % tends to be high. | Version B: data distributed in EQR range from 0.5 to 0.75: as consequence agreement % tends to be low. |
| --- | --- |

Figure 2. Illustration of the effect of differences in data distributions on the % agreements using 3 classes

Table 4 . Intercalibration Option 3 results: Agreement using 3 classes, arranged in increasing order.

| Abbreviation | Geographical Intercalibration group, Biological Quality Element and IC type (where appropriate) | Agreement % |
| --- | --- | --- |
| Random dataset |  | 45.0 |
| L CB Macrophy LCB 1 | Lake Central Baltic GIG Macrophytes Type LCB1 | 51.0 |
| L CB Macrophy LCB 2 | Lake Central Baltic GIG Macrophytes Type LCB2 | 53.8 |
| L CB Phyto | Lake Central Baltic GIG Phytoplankton | 55.9 |
| C MED Benthic Inv | Coastal Mediterranean GIG Benthic invertebrates | 60.5 |
| L Alp Macrophy 3 | Lake Alpine Macrophytes Type LAL3 | 68.5 |
| C BAL Benthic Inv SE-FI B2 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B2 | 68.7 |
| C MED Macroalgae | Coastal Mediterranean GIG Macroalgae | 69.4 |
| C NEA Benthic 6 | Coastal North East Atlantic GIG Benthic invertebrates (6 methods) | 70.2 |
| L Alp Macrophy 4 | Lake Alpine Macrophytes Type LAL4 | 71.4 |
| C NEA Benthic 5 | Coastal North East Atlantic GIG Benthic invertebrates (5 methods) | 76.3 |
| C BAL Benthic SE-DK | Coastal Baltic GIG (Sweden-Denmark) | 80.4 |
| C BAL Benthic SE-FI B3 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B3 | 83.3 |
| C BAL Benthic SE-FI B0 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B0 | 85.8 |
| Estimated maximum obtainable value |  | 85.0 |

Results of Agreement using 3 classes (see Figure 1):

- Range from 51% agreement (Lake Central Baltic GIG comparison of macrophyte assessment methods) to 86% (Coast Baltic GIG bilateral comparison of benthic fauna assessment systems of Sweden and Finland);

- Most of the results occur in range from 60% to 80% agreement;

- For comparison, a randomly distributed dataset showed 45% agreement (see Fig 1).

- The upper limit of the agreement cannot be expected to be 100% due to inherent uncertainty of the national biological assessment methods; the maximum obtainable level was estimated to be at a level of ca. 85%.

Figure 3. Comparison of the agreement (% of agreement) between Member States classification methods in different GIGs and quality elements using 3 classes (High, Good, and less than Good), arranged in increasing order (dark blue - Lake GIGs, light blue – Coast GIGs, yellow - random dataset and maximum expected level of agreement with uncertainty of method 10% )

This indicator is especially sensitive for how data are distributed over the quality range (see Figure 2). A relatively high agreement is obtained when a large proportion of the data is located in the moderate/poor/bad status area (this is the case in Coast Baltic GIG Benthic invertebrate methods comparison); a lower agreement is obtained when data is distributed mainly in high-good zone (as is the case in Lake Alpine Macrophytes Type LAL3).

3. Level of Agreement using 5 classes

- This indicator reflects a degree of consensus (level of agreement) between two or more assessment systems expressed as the percentage of cases where two methods give the same class classification result;

- Comparison using all 5 quality classes - two methods give compliant results if both indicate high status, or good status, or moderate status, or poor status, or bad status.

- The main advantage of this indicator is that it is less dependent on how the data is distributed over the quality range, ensuring better comparability between GIGs.

- A drawback if this indicator is that also disagreements in the lower quality classes will affect the results.

Table 5. Intercalibration Option 3 results: Agreement using 5 classes, arranged in increasing order.

| Abbreviation | Geographical Intercalibration group, Biological Quality Element and IC type (where appropriate) | Agreement % - 5 classes |
| --- | --- | --- |
| Random dataset |  | 22 |
| L CB Macrophy LCB 1 | Lake Central Baltic GIG Macrophytes Type LCB1 | 33.9 |
| L CB Macrophy LCB 2 | Lake Central Baltic GIG Macrophytes Type LCB2 | 33.8 |
| L CB Phyto | Lake Central Baltic GIG Phytoplankton | 38.7 |
| C BAL Benthic SE-FI B2 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B2 | 52.5 |
| L Alp Macrophy LAL3 | Lake Alpine Macrophytes Type LAL3 | 54.6 |
| C MED Macroalgae | Coastal Mediterranean GIG Macroalgae | 57.7 |
| C MED Benthic | Coastal Mediterranean GIG Benthic invertebrates | 57.9 |
| C BAL Benthic SE-FI B3 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B3 | 61.9 |
| C BAL Benthic SE-FI B0 | Coastal Baltic GIG Benthic invertebrates (Sweden-Finland) type B0 | 65.4 |
| C NEA Benthic 6 | Coastal North East Atlantic GIG Benthic invertebrates (6 methods) | 66.7 |
| L Alp Macrophy LAL4 | Lake Alpine Macrophytes Type LAL4 | 71.4 |
| C BAL Benthic SE-DK | Coastal Baltic GIG (Sweden-Denmark) | 71.7 |
| C NEA Benthic 5 | Coastal North East Atlantic GIG Benthic invertebrates (5 methods) | 72.8 |
| Estimated maximum obtainable value |  | 73.0 |

Results of Agreement using 5 classes (see Figure 2):

- Range from 34 % agreement (Lake Central Baltic GIG comparison of macrophyte assessment methods) to 73% (Coast NEA GIG comparison of 5 benthic fauna assessment systems);

- Most of the results occur in range from 50% to 70% agreement;

- Agreement % calculated using randomly distributed dataset (22% agreement) was included for comparison.

- The upper maximum level of agreement (presuming 10 % uncertainty in the classification and intercalibration results), give a potential value of ca. 73%.

Figure 4. Intercalibration Option 3 results: agreement % using 5 classes of the GIG methods comparisons, arranged in increasing order (dark blue - Lake GIGs, light blue – Coast GIGs, yellow -random dataset and maximum expected level of agreement with uncertainty of the methods 10%)

4. Setting of border lines for Option 3 acceptance criteria

The most important task is to set the level of sufficient comparability – which level is acceptable and which is not acceptable for the various criteria. At the same time it is important to ensure comparability between performance of IC Option 2 and Option 3. So two approaches were developed in order to set acceptance criteria which are comparable to IC Option 2.

The work included following steps:

1. Selection of appropriate comparability indicator - the main criterion recommended to be used is the absolute average class difference due to following characteristics

- This indicator reflects to which extent classification systems give different results;

- Value is not sensitive for the data distribution over the quality range;

- Indicator shows also the cases of “strong misclassification” - when the results differ by two or more quality classes;

- Indicator enables comparison with Intercalibration Option 2 performance (see the next point).

2. Setting of the level of sufficient comparability.

1st approach: This estimate was obtained using a randomised dataset with the same level of uncertainty (i.e. band of acceptability of 10%) as accepted for the Option 2 comparisons; more detailed explanations will be provided for the next ECOSTAT meeting. From this analysis it was proposed to set the boundary for acceptability at half class (0.5) difference (so the results above 0.5 class difference are considered as not satisfactory).

2nd approach – calculation Option 3 acceptance criteria using Option 2 datasets

The levels of comparability in the Option 3 datasets were compared with already agreed Option 2 results.

      - The Alpine phytoplankton group has compared and harmonized three phytoplankton assessment indices using IC Option 2 approach (comparison of national metrics using IC Common Metrics);

      - This GIG followed agreed principles, all class boundary values fell in “±0.05 EQR harmonisation band” and was agreed by ECOSTAT as acceptable;

      - Alpine GIG dataset allowed calculating “Option 3” indicators for 3 national methods and two IC common lake types, resulting in 6 values for each indicator.

Figure 4 shows the values of absolute average class differences both for Option 2 and Option 3 datasets:

- Green colour: results from Option 2 datasets (already agreed and comparability demonstrated via Option 2 “acceptance band”);

- Blue colour: results from Option 3 datasets (subjected to the assessment of sufficient/ insufficient comparability);

- Red line:  the highest Option 2 results were used to set the level of sufficient comparability for Option 3 performance in order to ensure the comparability between different IC options.

Figure 4 . Absolute average class difference of the GIG methods comparisons, arranged in increasing calculated on Option 3 datasets (blue colour) and Option 2 datasets (green colour). The possible criteria for sufficient comparability is proposed to be less than a half class (0.5) difference (red line).

From this analysis it is proposed to set the boundary for acceptability at half class (0.5) difference (so the results above 0.5 class difference are considered as not satisfactory) which was in line with the results obtained using the 1st approach.

So both approaches (data analyses using randomized dataset and IC Option 2 dataset) gave the same results: The possible criterion for sufficient comparability is proposed   absolute average class difference less than 0.5 quality classes.

Actually comparability was demonstrated using option 3.

Finland decided to withdraw their method from the intercalibration. Other MS have used Option 2 for the comparison, so no more explanations needed

The German method was not included in the comparability analysis

The Belgian and Dutch methods were not included in the comparability analysis

This estimate was obtained using a randomised dataset with the same level of uncertainty (i.e. band of acceptability’ of 10%) as accepted for the Option 2 comparisons; more detailed explanations will be provided for the next ECOSTAT meeting

This estimate was obtained using a randomised dataset with the same level of uncertainty (i.e. band of acceptability’ of 10%) as accepted for the Option 2 comparisons; more detailed explanations will be provided for the next ECOSTAT meeting