Determination of the sum of nitrite and nitrate nitrogen, nitrate nitrogen and total nitrogen in water by automated analytical equipment - monitoring method

(fact sheet)

| ##### Water Category | Natural water, waste water |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Sum of nitrite and nitrate nitrogen, nitrate nitrogen and total nitrogen |  |  |
| Name of the method  | Determination of the sum of nitrite and nitrate nitrogen, nitrate nitrogen and total nitrogen in water by automated analytical equipment. |  |  |
| Method proposed by  | ###### Finland |  |  |
| Method commonly used in  | Finland |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Nitrate is reduced to nitrite by a copper-cadmium reductor column. Nitrate is determined by diazotising with sulphanilamide and coupling with N-(1-naphtyl)-ethylenediamine to form a reddish-purple azo dye that is measured at 550 nm. |  |  |  |
| When analysing total nitrogen samples are first oxidized with an alkaline persulfate mixture. The oxidation takes place in a closed vessel at 120°C and under pressure. The oxidation process converts all nitrogen-containing compounds to nitrate. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | EN ISO 13395:1996 |
| National Standard (yes/no) – if yes provide reference number |  |  | SFS 3029, SFS 3030 |
| National method (yes/no) – if yes provide reference number and details |  |  |  |
| Published in literature (yes/no) – if yes provide reference details |  |  |  |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  |  |  |
| Expert e-mail address |  |  |  |
| URL: |  |  |  |
| Telephone number |  |  |  |