Fresh water algal growth inhibition test with Scenedesmus subspicatus and Selenastrum capricornutum  - monitoring method

(fact sheet)

| ##### Water Category | Lake, River, Ground water, Effluents |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Setting the environmental quality standard chronic toxicity data |  |  |
| Name of the method  | Fresh water algal growth inhibition test with Scenedesmus subspicatus and Selenastrum capricornutum |  |  |
| Method proposed by  | ###### Finland |  |  |
| Method commonly used in  | Finland |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| This standard specifies a method for the determination of the toxic effects of chemical compounds on growth of planktonic freshwater algae. The test can be used for readily water-soluble substances which are not significantly degraded or eliminated from the test system. Monospecific strains are cultured for several generations in a defined medium containing a range of concentrations of test substance prepared by mixing appropriate quantities of nutrient concentrate, water, test substance stock solutions, and an inoculum of exponentially growing algal cells. The test solutions are incubated for a minimum period of 72 h, during which the cell density in each is measured at least every 24 h. Inhibition is measured as a reduction in growth rate relative to control cultures grown under identical conditions. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – yes if yes provide reference number |  |  | EN 28692:1993 ISO 8692:1989 |
| National Standard (yes/no) – yes if yes provide reference number |  |  | SFS-EN 28692:1993 and SFS 5072 |
| National method (yes/no) – if yes provide reference number and details |  |  |  |
| Published in literature (yes/no) – if yes provide reference details |  |  |  |
| Reference material |  |  |  |
| EN 28692:1993; Water quality –Fresh water algal growth inhibition test with Scenedesmus subspicatus and Selenastrum capricornutum SFS 5072: 1986; Water quality – Toxicity test with pure culture of algae ( in Finnish) |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Marja Ruoppa |  |
| Expert e-mail address |  | marja.ruoppa@ymparisto.fi |  |
| URL: |  |  |  |
| Telephone number |  | + 358 9 40300403 |  |