Submerged macrophytes in lakes - monitoring method

(fact sheet)

| ##### Water Category | Lakes |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Macrophytes and phytobenthos– composition and abundance |  |  |
| Name of the method  | Submerged macrophytes in lakes |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Occurrence of aquatic macrophytes is analysed in depth intervals of 0.25 (0.5) m. At each depth interval, starting from 0-0.5 m depth, the vegetation is analysed in at least 10 quadrates (0.5 x 0.5 m). In each quadrate species presence and cover are recorded. Observations are made by means of a viewing tube with clear perspex base. In deep water the most optimal way of surveying a lake bottom is by divers but when excluded the survey should be based on a frequency analysis with rake. The quadrates can be placed at random on chosen sites representing a part of a lake, or along transects perpendicular from the bank and out to the depth limit of vegetation. The number of sites or transects for each lake should be adjusted according to lake size. The deviation between observed and reference conditions is measured with a Trophic Ranking Score for each taxon and a mean score for the site or lake. The results are expressed with quality classes when it is relevant. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Tests of monitoring method carried out in national reference lakes of Sweden. Comparison of lake status is described in “Environmental quality criteria. Lakes and watercourses.“ Swedish Environmental Protection Agency (SEPA) Report 5050, 2000. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | no |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no |
| National Standard (yes/no) – if yes provide reference number |  |  | no |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes. SEPA.www.environ.se Manual of methods. Version 1:1:2002-04-08 |
| Published in literature (yes/no) – if yes provide reference details |  |  | no |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Consistent with WFD if/when a data base of lake vegetation is completed |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Berta Andersson |  |
| Expert e-mail address |  | Berta.Andersson@ma.slu.se |  |
| URL: |  |  |  |
| Telephone number |  |  |  |