ANC (Acid neutralizing Capacity) - monitoring method

(fact sheet)

| ##### Water Category | Rivers and lakes |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Acidification status |  |  |
| Name of the method  | ANC (Acid neutralizing Capacity) |  |  |
| Method proposed by  | ###### Norway |  |  |
| Method commonly used in  | Norway, Sweden, Finland, UK, Italy |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| ANC is a commonly used method for assessing the acidification status of rivers and lakes and for dose/response relationship between aquatic biota and acidification status. |  |  |  |
| ANC is the basis for calculation of critical loads and levels of of sulphur and nitrogen to |  |  |  |
| surface waters. |  |  |  |
| ANC is calculated as (Base Cations) - (Acid Anions), equal to (Ca2+ + Mg2+ + Na+ + K+) - (SO42- + NO3- + Cl-), where the units are µeq/L. Due to charge balance in water, ANC can also be calculated as HCO3- + Organic anions-)-  (H+ - Aln+). This second method has more analytical problems with each of the parameters than the first method. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Database with best existing sites is available at NIVA, Norway for a number of water body types for both lakes and rivers. Reference conditions can be described as soon as a typology system is in place. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes, as soon as a typology system is in place. |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no |
| National Standard (yes/no) – if yes provide reference number |  |  | no |
| National method (yes/no) – if yes provide reference number and details |  |  | no |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes, see below |
| Reference material |  |  |  |
| Reuss, J. O. and Johnson, D. W. 1986. Acid Deposition and the Acidification of Soils and Waters. Ecological Studies. Springer Verlag, New York 119 pp.. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| ANC is an easy method for assessing the expected biological conditions for biota attributable to stress from acidification (mostly gill-breathing animals). |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Brit-Lisa Skjelkvaale |  |
| Expert e-mail address |  | brit.skjelkvaale@niva.no |  |
| URL: |  | www.niva.no |  |
| Telephone number |  | 47 - 22185209 |  |