Phytoplankton chlorophyll a - assessment method

(fact sheet)

| ##### Water Category | Marine coastal waters /Baltic Sea |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Biomass of phytoplankton |  |  |
| Name of the method  | Phytoplankton chlorophyll a |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Sampling and laboratory : see HELCOM |  |  |  |
| Assessment: The values presented refer to surface waters at depths of 0–20 metres. Assessment of current conditions : The classification is based on chlorophyll-a levels measured along the Swedish coast during the period around 1990. The class limits have been set so that each class includes one-fifth of the measured values. Deviations from reference values : The quotient between measured level and reference value is used here to indicate the extent to which the chlorophyll-a level varies from natural, pristine conditions. A quotient of 1 (measured transparency/reference value) is used as the boundary between classes 1 and 2. The boundary between classes 4 and 5 has been set so that class 5 includes ten percent of the levels measured during 1988–90 in waters along the Swedish coast. In general, values in class 5 indicate pollution from a local point source. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference values are estimates of the concentrations to be expected in waters not significantly affected by human activity. Separate reference values are provided for each type of area. In the Bothnian Bay and the Bothnian Sea, chlorophyll-a levels have not undergone any significant change in recent decades. For that reason, the reference values chosen for those areas consist of median levels measured during 1987–91 in areas that have not been affected by local sources of pollution. The reference values for the Baltic proper are based on historical data and constitute estimated average levels for the period around 1950. For Skagerrak/Kattegat, they are based instead on averages during 1979–93; those figures have been adjusted downwards on the basis of currently available information. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes, but need further development. |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  |  |
| National Standard (yes/no) – if yes provide reference number |  |  |  |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Swedish Environmental Protection Agency 2000. Environmental Qality Criteria – Coasts and Seas. Report 5052. http://www.internat.naturvardsverket.se/index.php3, Legislation/guidelines, Environmental Quality Criteria, Coasts and Seas |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Yes. But development needed i.e. for setting class boundaries between status classes and adjusting typology. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Tove Lundeberg |  |
| Expert e-mail address |  | tove.lundeberg@naturvardsverket.se |  |
| URL: |  |  |  |
| Telephone number |  |  |  |