Temperature - monitoring method

(fact sheet)

| ##### Water Category | Coastal waters / Baltic Sea |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Thermal conditions |  |  |
| Name of the method  | Temperature |  |  |
| Method proposed by  | ###### HELCOM |  |  |
| Method commonly used in  | All HELCOM Contracting Parties |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Temperature is to be determined by temperature sensors as on a CTD (as well as in a flow-through system onboard ships-of-opportunity or on an automatic station) or by reversing thermometer. The measurements can be done from different platforms, i.e. research vessels, ships-of-opportunity and fixed platforms. If CTD system is used, both temperature profile and temperature values at the moments when water samplers are tripped should be stored. Handling and calibration of CTD is described in Technical Annex I. Reversing thermometers should be calibrated using standard facilities at least once every two years. Mercury thermometer readings should be corrected using the Sverdrup formulae, as given by La Fond (1951). If digital reversing thermometers are used, one should follow the manufacturer's recommendations. Unattended measurements require special consideration according to (UNESCO, 1993; Rantajärvi and Leppänen, 1994). The required accuracy for thermometers and sensors used is 0.1°C in coastal waters. |  |  |  |
| More detailed description is found in the HELCOM COMBINE Manual: http://www.helcom.fi/Monas/CombineManual2/CombineHome.htm |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | Standard method for HELCOM monitoring |
| National Standard (yes/no) – if yes provide reference number |  |  |  |
| National method (yes/no) – if yes provide reference number and details |  |  |  |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes: http://www.helcom.fi/Monas/CombineManual2/CombineHome.htm |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Juha-Markku Leppänen |  |
| Expert e-mail address |  | juha-markku.leppanen@helcom.fi |  |
| URL: |  |  |  |
| Telephone number |  |  |  |