Littoral waters - monitoring method

(fact sheet)

| ##### Water Category | Coastal waters |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Nutrients Concentration in seawater |  |  |
| Name of the method  | Method for monitoring littoral waters |  |  |
| Method proposed by  | ###### Spain |  |  |
| Method commonly used in  | Spain (Catalonia) |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The monitoring and control of the quality of the coastal waters has been designed in order to study the whole littoral zone and its peculiarities. Consequently, monitoring networks have been defined in three different fields: near field, mid field and far field. These fields have well-differentiated masses of water and are subjected to various degrees of continental impact. Physical and chemical characterisation of sampling sites is conducted in situ (temperature, pH, salinity and dissolved oxygen) and in the laboratory (nutrients: nitrates nitrites, ammonium, phosphates, silicates, chlorophyll a, BOD5 and microbiology). Observing the obtained data, the presence of freshwater in coastal water and its origin (natural, anthropogenic or mixed) can be detected. As a result, a continental alteration degree of an specific coastal area can be given. In addition, statistical data analysis (multivariate analytical methods) allows the establishment of quality gradients along the coast and identifies the factors that determine this quality. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| This method has been used in order to (i) describe the nature and the state of the littoral waters in relation to their trophic characteristics; (ii) to assess the effects of the continental deposits in the receiving littoral marine medium and (iii) to evaluate the quality of the littoral waters and know their distribution along the littoral in Catalonia. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no |
| National Standard (yes/no) – if yes provide reference number |  |  | no |
| National method (yes/no) – if yes provide reference number and details |  |  | no |
| Published in literature (yes/no) – if yes provide reference details |  |  | No (in scientific journals), yes in technical reports |
| Reference material |  |  |  |
| Agencia Catalana de l’Aigua & Institut de Ciències del Mar, 2002. Method for monitoring littoral waters. Technical report. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Jordi Camp |  |
| Expert e-mail address |  |  |  |
| URL: |  |  |  |
| Telephone number |  | 00-34-93-230-95-00 |  |