Benthic invertebrate fauna (transitional and coastal waters) - monitoring method

(fact sheet)

| ##### Water Category | Transitional and coastal waters |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of benthic invertebrate fauna |  |  |
| Name of the method  | Benthic invertebrate fauna |  |  |
| Method proposed by  | ###### UK |  |  |
| Method commonly used in  | UK |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Assessment of the benthic invertebrate fauna is a standard technique applied widely in transitional and coastal waters both to assess impacts and to describe environmental status. The techniue is employed both subtidally and intertidally in transitional and coastal waters. Standard methods exist for a number of sampling techniques including grabs (Day and Van Veen) and corers (Hand and Box). Standard methods also exist for extracting and processing fauna recovered using these samplers and for estimating the total number of species, abundance and biomass. The measures generated are used in a variety of ways to meet the requirements of the survey or monitoring objectives. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| The measures of benthic invertebrate community structure are not compared to reference conditions at present. Research is ongoing on how measures derived using this method can be used for the purposes of the WFD. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | Yes – method used by ICES. |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes – Environment Agency National Marine Procedures Manual (methods B101-104, C101-102 and D101-102). |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Baker, J.M., & Wolff, W.J., eds. (1987). Biological Surveys of Estuaries and Coasts. Estuarine and Brackish Water Sciences Association Handbook. Cambridge University Press, 449pp. Barnett, B.E. (1993). National Standard Methodology for Marine Macrofaunal Benthic Sampling. III. Guidelines for Sample Pre-treatment for Marine Benthic Invertebrate Samples. NRA Internal Document. Holme, N.A., & McIntyre, A.D., eds. (1984). Methods for the Study of Marine Benthos. 2nd edn. pp1-26. Blackwell Scientific, Oxford. NPMMG (1999). UK National Marine Monitoring Programme Green Book. NRA (1993). Standard Methodologies. Assessment of Estuarine/Marine Water Quality Using Benthic Macroinvertebrates. National Rivers Authority, Anglian Region. OSPAR (1997). JAMP Eutrophication Monitoring Guidelines: Benthos. Oslo and Paris Commissions Joint Assessment and Monitoring Programme (9/6/97). |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The technique generates estimates of the benthic invertebrate fauna measures required by the WFD. At present the measures generated are not referenced based not used in a classification of environmental status. These issues are being addressed as part of current R&D. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Alastair Ferguson |  |
| Expert e-mail address |  | Alastair.ferguson@environment-agency.gov.uk |  |
| URL: |  | www.environment-agency.gov.uk |  |
| Telephone number |  | 020 8310 5500 |  |