Organotin - monitoring method

(fact sheet)

| ##### Water Category | Coastal/Transitional water |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Contaminants |  |  |
| Name of the method  | Organotin determination in sediments |  |  |
| Method proposed by  | ###### Netherlands |  |  |
| Method commonly used in  | Netherlands,… |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Determination of butyl- and phenyltin compounds in sediment and suspended matter with GC-AED detection: extraction of alkyl tin compounds from sediments can be accomplished by leaching with acidified (cosolvent assisted) aqueous mixtures followed by determination of the alkyltin compounds in the leachate. After extraction of the wet sediment with methanol, acetic acid and hexane the tri-, di- and monoalkyl tin compounds can be directly ethylated in a (preferably freshly prepared) aqueous solution of sodiumtetraethylborate (NaB(Et)4). A typical concentration for the reagent is 5 % (NaB(Et)4) from wich about 4 ml reagent is added. The very rapid derivatisation takes place at a pH between 4 and 6. The alkyl tin compounds are extracted to an organic phase for off-line cleanup followed by GC analysis. The alkyl tin compounds migrate to the organic phase (for instance hexane) after addition of (extra) hexane which can be removed through centrifugation. The ethylated alkyl tins, including both phenyl and butyltins, are prepared for GC analysis after subsequent cleanup over Al2O3 and solvent evaporation. Quantification should be done by using internal standards. Internal standards must be present in all samples, standards, and blanks at identical levels. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Cai, Y., Rapsomanikis, S., Andreae, M. O. (1993) Determination of butyltin compounds in sediment using gas chromatography-atomic absorption spectrometry: comparison of tetrahydroborate and sodium tetraethylborate derivatisation methods. Anal. Chim. Acta, 274, 243-251. Smedes, F., A.S. de Jong and I.M. Davies (2000). Determination of (mono, di- and) tributyltin in sediments. Analytical methods. J. Environ. Monit. 2, 541-549. Wilken, R.D., Kuballa, J. and Jantzen, E. (1994) Organotins: their analysis and assesment in the Elbe river system, Nothern Germany, Fres. J. Anal. Chem., 350, 77-84. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | yes |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no |
| National Standard (yes/no) – if yes provide reference number |  |  | no |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes, RIKZ method Standard Operating Procedure A 645 |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes, see above |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Consistent |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | A.S. de Jong |  |
| Expert e-mail address |  | A.S.dJong@rikz.rws.venw.nl |  |
| URL: |  | www.rikz.nl |  |
| Telephone number |  | +3150 5331307 |  |