Quantitative sampling of fish with electricity - monitoring method

(fact sheet)

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Fish species composition, abundance and age structure |  |  |
| Name of the method  | Quantitative sampling of fish with electricity |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The method provides an absolute estimate of species occurrence, abundance, biomass and size structure of fish assemblages at selected localities (sites) in wadable rivers (maximum depth 0,7 m). Sampling of fish is performed with generator- or battery-powered electrofishers, producing smooth direct current (DC) or pulsating direct current (PDC), and successive removal technique (multiple fishing). To obtain absolute estimates of population density (numbers per 100 m2) and efficiency of the sampling effort for occurring species, each site is sampled with at least three removals (efforts). Measurements of the catch includes measurement of fish length and weight (individually for large fish and in groups for small fish). After fishing and measurements all fish are released at the capture site. Population estimates and sampling efficiency are normally calculated using the Zippin method. If the catchability is to poor, as for sculpins, the mean catchability values for each species (calculated from the database) are used for population estimates. The number of sites per assessment unit are at least three. Prefered sampling season is early autumn (Aug-Sept). |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Fish data collected with this method are continuously compiled in the Swedish Electrofishing RegiSter (SERS), which at present contains more than 22 000 electrofishing occassions and 9 000 fishing sites (www.fiskeriverket.se). The database consist of five ”files”: LOCALITIES, contains geographical data for the fishing sites, FISHING, contains data of the electrofishing performance, CATCH, contains data about the catch; species occurrence and abundances, LENGTH, contains all the fish lengths from the catch, CHEMISTRY, contains data of the water chemistry (if such a sample has been taken). The files are connected with the x- and y-coordinate (Swedish coordinate system, RAK). The data in the database will be used in the revision of the Swedish fish index, including values for type specific reference states. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Not exactly, but the Swedish fish index will be revised. |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | Yes, it is included in the “Swedish handbook of environmental monitoring” |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes, it is included in the “Swedish handbook of environmental monitoring” |
| Published in literature (yes/no) – if yes provide reference details |  |  | No |
| Reference material |  |  |  |
| Bohlin, T, S. Hamrin, T.G. Heggberget, G Rasmussen & S.J. Saltveit. 1989. Electrofishing – theory and practice with special emphasis on salmonids. Hydrobiologia 173: 9-43. Zippin, C. 1958. The removal method of population estimation. J. Wildlife Management 22 : 82-90. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The method collects data on species composition and quantitative data on abundance (numbers per 100 m2). Age and age structure can be determined from length-frequency data, scales or other body parts. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Björn Bergquist |  |
| Expert e-mail address |  | bjorn.bergquist@fiskeriverket.se |  |
| URL: |  |  |  |
| Telephone number |  | +46 8 620 445 |  |