Biological GQA (RIVPACS) - monitoring method

(fact sheet)

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of benthic invertebrate fauna |  |  |
| Name of the method  | Biological GQA (General Quality Assessment) classification |  |  |
| Method proposed by  | ###### United Kingdom |  |  |
| Method commonly used in  | United Kingdom, Isle of Man |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Samples must be collected in spring and autumn and analysed by standard methods and the analyses must be audited (see Murray-Bligh 1999a & b). Two indices are used for this classification are the average BMWP-score per taxon (ASPT) and the number of BMWP-scoring taxa (N-taxa, i.e. the number of major groups used in the scheme). The BMWP-score for each taxon (family) is a numerical value based on the tolerance to organic pollution of its most tolerant species. The computer model RIVPACS III, is used to predict the ASPT and N-taxa that would be expected at the site if its water quality was good (referecne condition). The observed ASPT and N-taxa are divided by the predicted values, to express them as fractions of what they would be if water quality was good. These fractions are called EQIs. The classification is based on both EQI ASPT and EQI N-taxa. The grade indicated by the EQI representing the poorest quality is the grade allocated to the site. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference condition is based on 614 sites in Britain and 70 sites in Northern Ireland which are considered to be good quality. Reference conditions for other sites are predicted by associating physical and physico-chemical parameters to those observed at reference sites, and similarity of the other sites to every site type covered by the reference sites. Observations are compared to reference conditions by expressing observed biotic indices (ASPT and N-taxa) as fractions of the values predicted in reference conditions (EQIs, as explained above). |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Biological Monitoring Working Party (1978, unpublished report) Final report of the Biological Monitoring Working Party: assessment and presentation of the biological quality of rivers in Great Britain. London: Department of the Environment Water Data Unit. Clarke, R.T., R. Cox, M.T. Furse, J.F. Wright and D. Moss, 1997 RIVPACS III+ River InVertebrate Prediction and Classification System with error assessments User Manual. Environment Agency Technical Report E26. Bristol, Environment Agency. Cox, R., J.F. Wright, M.T. Furse, D. Moss, 1995 RIVPACS III (River InVertebrate Prediction and Classification System) User Manual. NRA R&D Note 454. Bristol, National Rivers Authority. Hawkes, H.A. (1997) Origin and development of the Biological Monitoring Working Party score system. Technical Note. Water Research, 32 (3), 964-968. Environment Agency, 1996 The quality of rivers in England and Wales (1990 to 1995) A report by the Environment Agency. Bristol, Environment Agency. Environment Agency, 1997 Assessing Water quality - General Quality Assessment (GQA) scheme for Biology. Fact Sheet. Bristol, Environment Agency. Environment Agency, 1997. The quality of rivers and canals in England and Wales 1995. Bristol: Environment Agency. Environment and Heritage Service 1997 River Quality in Northern Ireland 1995. Belfast: Environment and Heritage Service, Department of the Environment for Northern Ireland. Murray-Bligh, JAD 1996 Procedure for collecting and analysing macro-invertebrate samples for GQA surveys. Quality Management Systems for Environmental monitoring: Biological Techniques, BT002. (Version 2.1, February 1996). Bristol, Environment Agency. Murray-Bligh, JAD 1999a Procedures for collecting and analysing macro-invertebrate samples. Quality Management Systems for Environmental monitoring: Biological Techniques, BT001. (Version 2.0, 30 July 1999) Bristol, Environment Agency. Murray-Bligh, JAD 1999b Procedure for quality assurance for RIVPACS compatible macro-invertebrate samples analysed to the taxonomic level needed for the BMWP-score system. Quality Management Systems for Environmental monitoring: Biological Techniques, BT003. (Version 1.0, 3 August 1999) Bristol, Environment Agency. Murray-Bligh, J.A.D. (2000). Modifications to the methods used for sampling invertebrates from deep waters (BT001). National Biology Technical Group Working Document No. 38. (internal document). Wright, J.F., D.W. Sutcliffe & M.T. Furse (2000). Assessing the biological quality of freshwaters. RIVPACS and similar techniques. Ambleside: Freshwater Biological Association. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Other EQIs may need to be derived to cover a wider-range of environmental pressures than are detected by ASPT and N-taxa. EQI values need to be re-banded to produce a WFD-compliant classification scheme. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | John Murray-Bligh |  |
| Expert e-mail address |  | John.murray-bligh@environment-agency.gov.uk |  |
| URL: |  |  |  |
| Telephone number |  | +44 (0)1292 352 225 |  |