Nutrient determination - assessment method

(fact sheet)

| ##### Water Category | Coastal waters / Baltic Sea |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Chemical and physico-chemical elements supporting the biological elements |  |  |
| Name of the method  | Nutrient determination |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Sampling and laboratory : see HELCOM |  |  |  |
| Assessment: Assessments of nutrient levels in coastal and marine waters can be based partly on the supply of total nitrogen and phosphorus during both summer and winter, and partly on winter levels of phosphates and of nitrogen in ammonium, nitrates and nitrites. Nutrient levels refer to surface waters at depths of 0–10 metres. Classification of current conditions is based on nutrient levels measured in surface waters along the Swedish coast during several years around 1990. The class limits have been set so that each class includes one-fifth of the measured values. |  |  |  |
| Classification of deviation from reference values: The quotient between measured value and reference value is here used to indicate the extent to which nutrient levels vary from natural, pristine conditions. |  |  |  |
| A quotient of 1 (measured level/reference value) is used as the boundary between classes 1 and 2. The boundary between classes 4 and 5 has been set so that class 5 includes ten percent of the levels measured around 1990 in surface waters along the Swedish coast. In general, the levels of class 5 indicate pollution from a local point source. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Type-specific reference values are estimates of the levels to be expected in waters that have not been significantly affected by human activities. In the Bothnian Bay and the Bothnian Sea, nutrient levels in coastal waters have not undergone any significant changes in recent decades. For that reason, the reference values chosen for those areas consist of the median levels measured during 1987–91 in areas that have not been affected by local sources of pollution. Reference values for the Baltic proper are based on historical data and constitute estimated average levels for the period around 1950. With regard to the Skagerrak/Kattegat, the reference values chosen are averages for the period from 1979–93; these values have been adjusted downwards on the basis of currently available information. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes, but need further development |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  |  |
| National Standard (yes/no) – if yes provide reference number |  |  |  |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Swedish Environmental Protection Agency 2000. Environmental Qality Criteria – Coasts and Seas. Report 5052. http://www.internat.naturvardsverket.se/index.php3, Legislation/guidelines, Environmental Quality Criteria, Coasts and Seas |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Yes. But development needed i.e. for setting class boundaries between status classes and adjusting typology. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Tove Lundeberg |  |
| Expert e-mail address |  | tove.lundeberg@naturvardsverket.se |  |
| URL: |  |  |  |
| Telephone number |  |  |  |