QBR Index

(fact sheet)

| ##### Water Category | River |  |
| --- | --- | --- |
| ##### Water Framework Directive Quality element | Morphological conditions- Structure of the riparian zone |  |
| Name of the method  | QBR Index |  |
| Method proposed by  | ###### Spain |  |
| Method commonly used in  | Spain |  |
| Short description of the method (approx. 10 lines): |  |  |
| The index, named QBR, is based in four components of riparian habitat: Total riparian vegetation cover, Cover structure, Cover quality and Channel alterations. It also takes into account differences in the geomorphology of the river from its headwaters to the lower reaches. These differences are measured in a simple quantitative way. For index determination, the river is divided into two sections: the main channel and the riparian area. The former is subdivided into two: the area permanently covered with flowing water (which is not considered in the scoring process), and the channel zone between the permanent flowing reach and the bankfull state (see the figure in the upper part of the two-sided form sheet). Helophytes are commonly found in the zone between the instream channel and the bankfull, and are used in the index as an element to increase its ecological value. This index does not consider submerged macrophytes, because instream channel characteristics are not used. The QBR index is calculated in the field through a two-sided A4 page form that may be completed in 10 minutes. |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |
| The development of the QBR index included trials in four Mediterranean stream catchments in Catalonia, (NE Spain). Up to the moment the index has been used by different research teams and is currently tested in an intercomparative study of 12 watersheds along the Mediterranean Spanish coast. It is expected that changes to adapt the QBR index to other geographical areas in temperate and semiarid zones may be done without changes in the index rationale. It has been until now adapted to semi-arid streams of south-east Spain (Suárez-Alonso & Vidal-Abarca, 2000). The QBR index has also been tentatively used in Argentina (Toscano et al., in press), and adapted to be applied in high mountain areas of the Pyrenees (Carrascosa & Munné, 2000). |  |  |
| Reference conditions compliant with WFD (yes/no) |  | <to be completed> |
| Status of method |  |  |
| International Standard (yes/no) – if yes provide reference number |  | NO |
| National Standard (yes/no) – if yes provide reference number |  | NO |
| National method (yes/no) – if yes provide reference number and details |  | NO |
| Published in literature (yes/no) – if yes provide reference details |  | YES |
| Reference material |  |  |
| - A. Munné, N. Prat, C. Solà, N. Bonada & M. Rieradevall. A simple field method for assessing the ecological quality of riparian habitat in rivers and streams. QBR index Aquatic Conservation: Freshwater and Marine Ecosystems (in preparation) - Suárez-Alonso, M. L. Vidal-Abarca, M. R. (2000). “Aplicación del índice de calidad del bosque de ribera, QBR (Munné et al., 1998) a los cauces fluviales de la cuenca del río Segura.” Tecnología del Agua 201: 33-45. - Munné, A. Solà, C. & Prat, N. (1998a). “QBR: Un índice rápido para la evaluación de la calidad de los ecosistemas de ribera.” Tecnología del agua 175: 20-37. |  |  |
| Applicability of the method to WFD (Expert’s assessment of method): |  |  |
| There are five quality classes of riparian habitat which broadly correspond to those suggested in the Water Framework Directive. The QBR index uses simple measures, and may be helpful to define quality values in riparian habitats in absence of reference conditions. This index may be a useful tool for expert advising for the WFD. |  |  |

| Contact details |  |
| --- | --- |
| Name of contact/expert |  |
| Expert e-mail address |  |
| URL: |  |
| Telephone number |  |