Determination of the inhibitory effect of water samples on the light emission of Vibrio fischeri (Luminescent bacteria test) Parts 1, 2 and 3  - monitoring method

(fact sheet)

| ##### Water Category | Lake, River, Effluents |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Setting the environmental quality standard acute toxicity data |  |  |
| Name of the method  | Determination of the inhibitory effect of water samples on the light emission of Vibrio fischeri (Luminescent bacteria test) Part 1. Method using freshly prepared bacteria Part 2. Method using liquid-dried bacteria Part 3. Method using freeze-dried bacteria |  |  |
| Method proposed by  | ###### Finland |  |  |
| Method commonly used in  | Finland |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| ISO 11348 describes three methods for determining the inhibition of the luminescence emitted by the marine bacteria Vibrio fischeri. These methods are applicable to: waste water, aqueous extracts and leachates, fresh water (surface or ground water) or salt water and brackish water, especially the monitoring of changes in inhibition towards bacteria, and pore water. The inhibition of light emission by cultures of Vibrio fischeri is determined by means of a batch test. This is accomplished by combining specific volumes of the test sample or diluted sample with the luminescent bacteria suspension in a cuvette. The test criterion is the decrease of the luminescence, measured after 15 min and 30 min or optimally 5 min. The inhibitory effect of the water sample can be determined as LID or EC20 and/or EC50 values by means of a dilution series. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – yes if yes provide reference number |  |  | EN ISO 11348-1:1998 EN ISO 11348-2:1998 EN ISO 11348-3:1998 |
| National Standard (yes/no) – yes if yes provide reference number |  |  | SFS-EN ISO 11348-1:1999 SFS-EN ISO 11348-2: 1999 SFS-EN ISO 11348-3:1999 |
| National method (yes/no) – if yes provide reference number and details |  |  |  |
| Published in literature (yes/no) – if yes provide reference details |  |  |  |
| Reference material |  |  |  |
| EN ISO 11348 - Parts1, 2 and 3:1998; Water quality - Determination of the inhibitory effect of water samples on the light emission of Vibrio fischeri (Luminescent bacteria test) Part 1. Method using freshly prepared bacteria Part 2. Method using liquid-dried bacteria Part 3. Method using freeze-dried bacteria |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Marja Ruoppa |  |
| Expert e-mail address |  | marja.ruoppa@ymparisto.fi |  |
| URL: |  |  |  |
| Telephone number |  | + 358 9 40300403 |  |