Chironomid Pupal Exuviae Technique (CPET) for assessing lake status - monitoring method

(fact sheet)

| ##### Water Category | Lakes |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Benthic invertebrate fauna- Composition and abundance |  |  |
| Name of the method  | Chironomid Pupal Exuviae Technique (CPET) for assessing lake status |  |  |
| Method proposed by  | ###### United Kingdom.First suggested by Thienemann, Germany in 1910 |  |  |
| Method commonly used in  | United Kingdom |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Floating pupal exuviae (skins) are collected at the leeward shore of the lake with a 250 m mesh net on an extendable pole so it is not usually necessary to enter the water. Exuviae float for 1-2 days before sinking. Owing to the passive collection of discarded pupal exuviae there is little opportunity for the operator to influence the contents of the sample. Littoral and profundal species are represented in the sample. Four samples in different months between April and October have been found to provide 80-90% of the species obtained from sampling every month. Exuviae are identified to species level and the relative proportion of species present compared with expected proportions from suitable reference lakes is used to make a judgement on lake quality. The same can be achieved with generic data. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Temporal referencing has been performed by reconstructing conductivity, pH, nitrate and total phosphorus for sub-fossil larval chironomid remains in sediment core samples based on a modern training set of chironomid pupal exuvial data. Spatial referencing can be obtained by comparing observed to expected taxa after reference lakes have been chosen. Species indicator assemblages have been identified for different classes of lake based on their conductivity, this can also be done for pH and nutrients. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No - CEN standard awaited |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | No |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Ruse, L., 2002 Chironomid pupal exuviae as indicators of lake status. Archiv fűr Hydrobiologie 153 (3): 367-390. Ruse, L. 2002. Colonisation of gravel lakes by Chironomidae. . Archiv fűr Hydrobiologie 153 (3): 391-407 (example of reconstructing conductivity using training set). Wilson, R.S., 1996. A practical Key to the genera of pupal exuviae of the British Chironomidae. Wedmore: RS Wilson. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The CPET provides an assessment of lake quality based on the most ubiquitous and species-rich aquatic macroinvertebrate family. The method has been used to classify lake type and has been demonstrated to successfully model conductivity, pH and nutrients and to reconstruct historic conditions in lakes from sediment core samples. The method is reference based and a suitable classification scheme could be developed to conform to the requirements of the WFD. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Les Ruse |  |
| Expert e-mail address |  | les.ruse@environment-agency.gov.uk |  |
| URL: |  |  |  |
| Telephone number |  | 0118 9535939 |  |