Physical SEQ - monitoring method

Monitoring WG 2.7 - 29/05/2002

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Hydromorphological elements |  |  |
| Name of the method  | Physical SEQ (Quality Evaluation System) |  |  |
| Method proposed by  | ###### France |  |  |
| Method commonly used in  | France |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The method is based on the description of ecologically homogeneous river reaches with a field form that describes floodplain, banks and channel features. The features scores are used to calculated quality indices with mixed calculation rules using the most downgrading score for some features and weighted average for subordinate ones. The weight sets used for calculations are type-specific (a natural french river typology has been published in 1998, based on expert judgments). A diagnostic is given for the whole reach, for floodplain, banks and channel but also for landuse, liability to flooding, bankstructure, riparian vegetation, channel continuity and channel morphology. |  |  |  |
| A first version of this methodology, published in 1998, has been revised according some field tests in various river types in France and intercomparison field training (CEN\TC 230\WG2\TG5) in 2001. A new revised version will be published in 2002. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Agences de l’Eau 1 Ministère de l’Environnement, 1998 – SEQ-Physique (Version 0) : A system for the evaluation of the physical quality of watercourses. 15 p. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | <to be completed> |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | <to be completed> |
| National Standard (yes/no) – if yes provide reference number |  |  | <to be completed> |
| National method (yes/no) – if yes provide reference number and details |  |  | <to be completed> |
| Published in literature (yes/no) – if yes provide reference details |  |  | <to be completed> |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The methodology is compliant with WFD requirements. Nevertheless, some progress have to be made to improve the link between physical quality and biology as required in Annex V. Intercomparisons exercises with UK (RHS) and German (LAWA-von-Ort) methods allowed a better comparibility of assessements. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Stephane Stroffek |  |
| Expert e-mail address |  | Stephane.stroffek@eaurmc.fr |  |
| URL: |  |  |  |
| Telephone number |  | 33 4 72 71 26 03 |  |